ICECAPS Weekly Report

Dec. 12 – Dec. 18, 2012

Lance Roth – Phase II Tech

General Weather:

• 12/12 BKN020	91% RH	   0F 	20 knots	SN, BLSN

• 12/13 BKN010	90% RH 	   0F 	10 knots	-SN

• 12/14 BKN010	88% RH	 -5F 	  5 knots

• 12/15 FEW100	73% RH	-30F 	  5 knots

• 12/16 FEW100	73% RH	-30F	  2 knots

• 12/17 FEW080	68% RH 	-40F 	  5 knots

• 12/18 FEW100	80% RH	-20F	  3 knots	FzFg

Dataman Account:

• Normal

MWR:

• Normal

SODAR:

• 12/13 1628-1630 UTC checked and removed snow from enclosures.

POSS:

• Normal

MMCR:

• 12/15 1300 UTC Found in Standby Mode during receiver calibration “Error 7 occurred at

Open File +.vi:)Open File.”  Pressed continue and system began operating.

• 12/18 Found in Standby Mode after receiver calibration.  Rebooted system at 1245 UTC.

System found shut down at 2330 UTC.  System up and running at 2340 UTC

CAPABL:

• Normal.

MPL:

• 12/13 Found computer logged out.  Logged in and started MPL 1255 UTC.

VCEIL:

• Normal

IceCAM:

• Normal

PAERI:

• 12/09 Partial Blue Flags

• 12/16 Red Flags on Responsivity due to frost on mirror.

• 12/18 1238 UTC Red Flag.  Removed frost from fan causing issue with responsivity. 1255-1258 UTC

removed heavy frost from railing and mirror.

ASIA-A:

• 12/13 Connected new network cable to Arduwina at  UTC

IcePIC:

• 12/12 1415 UTC and 1700 UTC Took pictures of several broken snowflakes due to high winds.

• 12/13 1400 UTC Took pictures of plates.

• 12/18 Loaded pictures onto Dataman.

Radiosonde:

• 12/11 PM sonde failed due to High Winds.