ICECAPS Weekly Report

Dec. 19 – Dec. 25, 2012

Lance Roth – Phase II Tech

General Weather:

• 12/19 FEW100	74% RH	-30F 	10 knots

• 12/20 BKN100	75% RH 	-30F 	  0 knots	FzFg

• 12/21 OVC003	80% RH	-15F 	  5 knots	FzFg

• 12/22 BKN003	80% RH	-20F 	  5 knots	-SN

• 12/23 FEW030	60% RH	-40F	  2 knots

• 12/24 FEW080	61% RH 	-50F 	  5 knots	North Winds

• 12/25 FEW100	55% RH	-65F	10 knots	BLSN

Dataman Account:

• Normal

MWR:

• Normal

SODAR:

• Normal

POSS:

• Normal

MMCR:

• 12/21 Hung during Calibration.  Rebooted.  System started normally

• 12/25 Hung during Calibration.  Rebooted several times due to a constant 61 antenna position.

Normal Operation at 1414 UTC.

CAPABL:

• 12/19 Found Laser off.  Hit Emission button on remote at 1245 UTC.  System began running normal.

MPL:

• 12/21 Found computer logged out.  Opened computer and checked AMCS card 1530 UTC.

VCEIL:

• Normal

IceCAM:

• Normal

PAERI:

• 12/24 Partial Blue Flags

• 12/24 1327-1328 UTC, Covered hatch and removed frost from railing.

ASIA-A:

• Normal

IcePIC:

• 12/22 Took Pictures of snowflakes.  Trouble catching flakes on slide.

Radiosonde:

• Normal