# ICECAPS Weekly Report

Feb. 27 – Mar 06, 2013

Sven Lidstrom – Phase I Tech

General:

A cold week which created problems with the PAERI

• 03/01 Station experienced a power fluctuation as we were swapping generators at 1626 UTC.

Dataman Account:

• Normal.

MWR:

• Normal

SODAR:

• 03/02 Powerware Message popup window in the screen dumps due to power fluctuation.

POSS:

• Normal

MMCR:

• 03/01 Hung during calibration. First of the month it was the longer calibration cycle. Resetting the fault and then restarting failed. Rebooted system successfully and normal operations from 13.00 UTC.

CAPABL:

• 02/26 Noticed strange values and emailed Ryan Neely.

CAPABL restarted at 15.42 UTC with modified settings, V45 2.15 and FB limits 500 and 600.

MPL:

• Normal

VCEIL:

• Normal.

IceCAM:

• Normal.

PAERI:

• 03/01 Turned fan off to clean out snow from the PAERI fan intake at 13.13 UTC.

• 03/03 LW & SW Responsivity and Encoder Scene Confirm bars in red. Contacted Von.

Covered up mirror to clean off snow and rime around PAERI mirror at 15.45 - 15.50 UTC.

• 03/04 Only garabage data collected which might be due to the cold temperatures effecting the mirror movement. After contacting Von, unplugged and removed fan and covered hole with foam and plywood to insulate. 18. 20 UTC.

• 03/05 Insulating foam and plywood removed and fan put back in place at 17. 37 UTC

ASIA-A:

• Normal

IcePIC:

• No pictures taken.

Radiosonde:

• 02/28 Lost telemetry 28 min in to the launch and DAQ program got stuck.

• 02/29 Forgot to close off the evening launch in time.

• 03/05 Forgot to close off the launch in time.