# ICECAPS-ACE Weekly Report

April 15 – April 21, 2019

Nate Bowker

Plates and bullets juxtaposed.

General Notes:  Station opening activities are in full swing with the first LC-130 scheduled to arrive on Friday, 4-26.  Heavy equipment operation is occurring around station on a daily basis.

General Weather Observations:

| Date | Time (Z) | Wind | Weather | Visibility | Sky | Temp | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 4-15 | 12 | 21kt from 150 | DRSN FZFG | 800 | SCT100 | -19 |  |
| 4-16 | 12 | 18kt from 180 | DRSN FZFG | 800 | FEW030 | -23 |  |
| 4-17 | 12 | 30kt from 130 | BLSN | 100 | VV003 | -18 |  |
| 4-18 | 12 | 21kt from 180 | DRSN FZFG | 200 | VV006 | -23 |  |
| 4-19 | 12 | 20kt from 190 | FZFG | 600 | SCT100 | -23 |  |
| 4-20 | 12 | 12kt from 120 | BR | 1800 | BKN060 | -22 |  |
| 4-21 | 12 | 7kt from 197 |  | 9999 | FEW030 | -35 |  |

Significant Weather Observations:

- 4-16 ~0900 – 1330Z:  22-degree halo with infralateral arcs observed.

- 4-18 ~0700 – 1000Z:  22-degree halo observed.

- 4-18 ~1230 – 1400Z:  22-degree halo with infralateral arcs and a circumzenithal arc observed.

- 4-19 ~1600 – 1630Z:  22-degree halo observed.

- 4-20 ~1020 – 1200Z:  22-degree halo observed.

ICECAPS-ACE Data Management:

- 4-15: Data transfer completed at 0415Z.

- 4-16: Data transfer completed at 0415Z.

- 4-17: Data transfer completed at 0412Z.

- 4-18: Data transfer completed at 0415Z.

- 4-19: Data transfer completed at 0421Z.

- 4-20: Data transfer completed at 0415Z.

- 4-21: Data transfer completed at 0429Z.

MWR:

- Operating normally.

SODAR:

- 4-16 1000Z – 4-17 1300Z: No sodar JPGs due to my failure to leave software display maximized.

- 4-19 1640 – 1652Z:  Powered down amp and cleaned sodar dishes.

- Operating normally.

POSS:

- Operating normally

MMCR:

- 4-19 and 4-20:  Experienced radar calibration errors NetCDF_Mom and NetCDF_Spec.

- 4-21 1103 – 1321Z:  Performed full system reboot.  Required 5 attempts to achieve a successful startup.

- Temp (outside) value continues out of tolerance (at 30)

- Operating normally.

CAPABL:

- Instrument currently offline.

MPL:

- 4-15 1223-1246Z:  Monthly afterpulse calibration performed.

- Operating normally.

VCEIL:

- Operating normally.

IceCAM:

- Operating normally

PAERI:

- Operating normally.

TSI:

- Offline for winter season.

IcePIC:

- 4-19: One sampling session photographed.  Uploaded on 4-20.

- 4-20: One sampling session photographed and uploaded.

- 4-20: Microscope brought inside to thaw out.  Cleaned the interior of the photo booth of frost and snow.

- 4-21: Fabricated a simple mount for the microscope light source allowing for consistent low oblique transmitted illumination with incremental adjustability.

Radiosonde:

- 4-16 1200Z:  Sounding terminated prematurely (at ~12km) due to temperature sensor failure.

- 4-20 0000Z:  Sounding burst altitude was just over 14km.

- 4-20 1200Z:  Sonde damaged by snow removal activities when staged for launch outside of SMG.  Second sonde prepped and launched at 1135Z.

- Twice daily launches.

MASC:

- Instrument offsite for repair, will not return until summer 2019.

OPC:

- Operating normally.

CLASP:

- Instrument off site.

CPC:

- 4-17 1229Z: Emptied condensate reservoir.

- 4-18 1232Z: Changed outlet filter per 21-day replacement interval.

- Operating normally

Fractal frost and melting crystals.