# ICECAPS-ACE Weekly Report

September 6th – September 12th, 2022

Molly Peek

The first aurora of the season at Summit.

Photo taken by: Cody Spillane

General Notes:

This week at the Mobile Science Facility (MSF), all regular checks and projects continued as the facility is prepared for the winter crew. Daily radiosonde balloon launches and instrument checks were all successful. The MMCR-DM computer issue was resolved, and the instrument is once again fully operational. Network issues persisted, interrupting daily data transfers and offsite access to instruments. This issue is currently being resolved with IT. The MASC was taken offline due to a frayed wire, until a fix can be arranged. After abnormally warm temperatures last week, Summit Station saw gradual cooling and several fog and snow events.

General Weather Observations:

| Date | Time (Z) | Wind (kt) | Weather | Visibility | Sky | Temp (C) | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2022-09-06 | 12:00 |  |  | 1000 | OVC |  |  |
| 2022-09-07 | 12:00 |  |  | 9999 | OVC |  |  |
| 2022-09-08 | 12:00 |  | -SN | 800 | OVC |  |  |
| 2022-09-09 | 12:00 |  | BLSN, DRSN | 1000 | OVC |  |  |
| 2022-09-10 | 12:00 |  | DST FZFG | 9999 | SCT |  |  |
| 2022-09-11 | 12:00 |  |  | 9999 | SCT |  |  |
| 2022-09-12 | 12:00 |  |  |  |  |  |  |

Significant Weather Observations:

- Partial 22-degree corona 2022-09-08:1600z

- Full 22-degree corona 2022-09-10:1000z

- Full 22-degree corona 2022-09-11:1345z

MSF

- Operating normally

ICECAPS-ACE Data Management:

- Data transfer did not complete 2022-09-06

- Data transfer did not complete 2022-09-07

- Data transfer did not complete 2022-09-08

- Data transfer completed 2022-09-09:0453z

- Data transfer completed 2022-09-10:0516z

- Data transfer completed 2022-09-11:0358z

- Data transfer completed 2022-09-12:0346z

- Persistent data transfer issues are being resolved with Battelle IT

TSI:

- Operating normally

Radiosonde:

- Operating normally

MWR:

- Brightness temperatures logging irregularly starting 20220912

- Operating normally

SODAR:

- Amplifier turned off for cleaning 2022-09-11:1404-1415z

- Operating normally

POSS:

- POSACQ window found closed 20220912. Power cord found loose and reattached, and the software was restarted the same day.

- Operating normally

MASC:

- Taken offline until further notice because of frayed power cord. The de-ice is still connected to limit snow accumulation.

MMCR:

- MMCR-DM computer issues resolved

- Slightly low transmitter power

- MMCR- RADAR computer error 20220911. Resolved same day.

- Operating normally

MPL:

- Operating normally

CEILOMETER:

- Operating normally

PAERI:

- Expected to return in September

IceCAM:

- Operating normally.

IcePIC:

- No images captured

MAGIC CPC:

- Operating normally

MSF - OPC:

- Quicklooks pages are not accessible.

- Operating normally

AWO - OPC:

- Operating normally

SKYOPC:

- Offline

Biral PWS :

- Quicklooks pages accessible, data updating

- Operating normally

Flux Tower instrument suite:

- Operating normally

Radiometers:

- Operating normally